Pre-Flight Checklist

This is provided as an example and should be adapted to suit the UAS and operational team

| Personnel |  |  |
| --- | --- | --- |
| Pilot In Command |  |  |
| 2nd in Command |  |  |
| Public | Flight area clear / public informed of operations? | YES / NO |

| Mission Plan |  |  |  |
| --- | --- | --- | --- |
| Location |  | Aircraft |  |
| Planned Altitude |  | Planned Duration |  |
| Sensors |  |  |  |
| Mission Description |  |  |  |

| Item | Action | Result |  |
| --- | --- | --- | --- |
| Weather | Suitable for flight duration (wind and rain?) | YES / NO |  |
| Transmitter | Transmitter voltage sufficient? (>40%) | % / Volts |  |
| Tablet | Ground control tablet voltage sufficient? (>40%) | % / Volts |  |
| Flight battery | Battery ID |  |  |
|  | Flight battery sufficient charge? (>90%) | % / Volts |  |
| Airframe health | Structure ( YES / NO ) Propellers ( YES / NO ) Payload attached ( YES / NO ) SD Cards ( YES / NO ) No Loose Wiring ( YES / NO ) Motors Free to spin ( YES / NO ) |  |  |
| Protectors | All protection removed (e.g. gimbal clips, lens cap) | YES / NO |  |
| System | Connect batteries and turn on system | YES / NO |  |
| Ground station | RF connection(s) established and satisfactory? | YES / NO |  |
| GPS | GPS fix established (normal > 10 satellites) | YES / NO |  |
| Magnetometer | Calibrated for current location? | YES / NO |  |
| Altimeter | Altimeter near zero when on ground? | YES / NO |  |
| Transmitter | Check flight mode functions (including RTL) | YES / NO |  |
| Servo check | Check servos/actuators respond to pilot inputs | YES / NO |  |
| Flight plan | Upload and check auto flight plan | YES / NO |  |
| RTL | Check RTL altitude is higher than obstructions | meters |  |
| Payload | Check payload functioning? | YES / NO |  |
| People | Crew and public in suitable positions? | YES / NO |  |
| Air traffic | Visually check planned airspace is clear? | YES / NO |  |
| Go / No-go | Final check for those present Key personnel agree to start the mission | Ground station | Go / No-go |
|  |  | Pilot in command | Go / No-go |
| Time | Note take-off time | ____ : ____ |  |
| Power up | Start mission (either manual or auto) |  |  |
| Time | Note landing time | ____ : ____ |  |
| Flight battery | Note landed battery state | % / Volts |  |

Post-Flight Notes

Additional notes on mission implementation, success and issues